| # AgribotSim Integration & Verification Log | Abstract This project presents AgribotSim — a simulation-driven approach to intelligent crop monitoring using a Convolutional Neural Network (CNN) integrated with a MATLAB/Simulink-based robotic system. The robot navigates a grid-based environment, classifying tomato leaf health at each waypoint in real time using a trained MobileNetv2 model. Key implementation challenges, including class label misalignment between the CNN and Simulink, were systematically resolved using enumeration mappings and validation logic. The simulation prioritises realistic, sensor-driven decision-making, with visual debugging and validation mechanisms built in to confirm system reliability. Through modular design, structured data flow, and emphasis on scalability, AgribotSim lays the groundwork for future agricultural robotics — with potential for real-world deployment, multisensor integration, and expansion to other crop types. |
| --- | --- |

Final Submission Documentation for AGRIBOT_FINAL_SUB

## 1. Project Overview

This project demonstrates real-time AI-powered classification of tomato plant diseases within a Simulink-based robot simulation. A trained MobileNetV2 CNN model classifies leaf images into four categories: bacterial spot, fungal target spot, tomato mosaic virus, and healthy. The robot follows a waypoint path and simulates image analysis at each step, verifying predictions against expected labels.

## 2. CNN Development Summary

- Architecture: MobileNetV2, pre-trained and fine-tuned on a reduced subset of the PlantVillage dataset.
- Input Size: 224×224 RGB.
- Accuracy:
  - Validation: 99.2%
  - Test: 97.2% (batch result confirmed via external script).
- Efficiency: Training time reduced from ~116 hours to ~3.5 using MATLAB Parallel Computing Toolbox.
- Output: cleanTomatoNet.mat (DAGNetwork object).

## 3. Dataset & Preprocessing

Images were resized and renamed consistently using a MATLAB script. A labels.csv was generated to preserve class alignment post-renaming.
Dataset structure:
/resized_images/
 ├── Tomato___Bacterial_spot/
 ├── Tomato___Target_Spot/
 ├── Tomato___Tomato_mosaic_virus/
 └── Tomato___healthy/

## 4. Image Sorting & Label Fix

Issue: MATLAB dir loads files alphabetically (e.g., 10.jpg before 2.jpg).
Solution: Implemented numeric sort in the image loader using:
sort(str2double(extractBetween(files, '', '.')))

## 5. CNN Verification (MATLAB Script)

A batch image test was run pre-integration:
predictedLabels = classify(trainedNet, images);
accuracy = mean(predictedLabels == categorical(labels)');
Logged Accuracy: ~90.0% (realistic post-renaming). Output table included in report.

## 6. Simulink Integration & Architecture

The CNN model was integrated using the Predict block. Argmax logic converts softmax vector to class index.
Simulink uses a selector-based structure to step through the image batch based on robot movement.
A visual grid and logic panel aid debugging, with visualisation elements for class index display and validation.

## 7. Simulation Logic & Configuration

- Zig-Zag Navigation: Configured using waypoint_generation.m which defines a 6×6 scan pattern.
- Pause Logic: The robot pauses at each waypoint to simulate image capture.
- Grid Field: Red markers were initially used for visualisation but removed to reflect real-world design.

## 8. Prediction Validation Logic

A validation subsystem compares the CNN prediction to the expected class at each waypoint.
Categories: valid, invalid, systemOFF.
These checks are only active when the robot is “on-task,” ensuring accurate runtime validation.

## 9. Debugging & Engineering Decisions

Original red spheres were removed from logic to reflect real-world conditions. The simulation evolved to rely on sensor-based classification only.
This was a deliberate engineering decision to prioritise functionality over aesthetics.

## 10. Scalability Considerations

Future plans include:
- EKF-based multi-sensor fusion
- Webcam or live sensor data
- Classifier for other crops
- Improved simulation environments for scalability

## 11. Files & Reproducibility

All required files are available in: https://github.com/CarlaMED/AGRIBOT_FINAL_SUB
Includes: cleanTomatoNet.mat, saveWaypointImages.m, waypoint_generation.m, ClassLabel.m, Simulink models, config scripts, logs.

## 12. Toolboxes and setup:

• Deep Learning Toolbox

• Computer Vision Toolbox

• Simulink

• Image Processing Toolbox

• Parallel Computing Toolbox

• DPS Toolbox

• Simscape Electrical

• ImageMagick (used optionally to resize and rename images via command line)

## 13. References

MathWorks, 2023. ImageDatastore. [online] Available at: https://uk.mathworks.com/help/matlab/ref/imagedatastore.html [Accessed 16 June 2025].

MathWorks, 2023. MobileNet-v2 Pretrained Network. [online] Available at: https://uk.mathworks.com/help/deeplearning/ref/mobilenetv2.html [Accessed 16 June 2025].

MathWorks, 2023. Simulink. [software] The MathWorks, Inc. Available at: https://uk.mathworks.com/products/simulink.html [Accessed 16 June 2025].

MathWorks, 2023. Enumerations in Simulink. [online] Available at: https://uk.mathworks.com/help/simulink/ug/using-enumerated-types.html [Accessed 16 June 2025].

Hughes, D.P. and Salathe, M., 2015. An open access repository of images on plant health to enable the development of mobile disease diagnostics. arXiv preprint arXiv:1511.08060.

Simulink CNN Integration Summary

• CNN_Predictor subsystem includes Predict block, MATLAB Function (argmax), and display.

• Test harness isolates prediction logic.

• Label-to-index enum (ClassLabel.m) aligns outputs.

• A Selector block uses waypoint index to feed test images to the CNN in sync with robot motion.

System Layout Diagram (Simplified)

[testImage Selector] --> [Predict Block] --> [Argmax] --> [Label Display]

|                     ↘

[Image Index]        [Color Logic (future)]